The DVB-RCS MIB
draft-combes-ipdvb-mib-rcs-01.doc

Status of this Memo

By submitting this Internet-Draft, each author represents that       any applicable patent or other IPR claims of which he or she is       aware have been or will be disclosed, and any of which he or she       becomes aware will be disclosed, in accordance with Section 6 of       BCP 79.

This document may not be modified, and derivative works of it may not be created, except to publish it as an RFC and to translate it into languages other than English, or to extract section 3 as-is for separate use.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at
     http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at
     http://www.ietf.org/shadow.html

This Internet-Draft will expire on January 31, .

Abstractf

This document describes the MIB module for the Digital Video Broadcasting Return Channel via Satellite system (DVB-RCS). It defines a set of new MIB entities to characterise the behaviour and performance of network layer entities deploying DVB-RCS.

Table of Contents

1. Introduction	3

2. Conventions used in this document	4

2.1. MIB Module	4

3. Definitions	5

4. Security Considerations	65

5. IANA Considerations	65

6. Conclusions	Error! Bookmark not defined.

7. Acknowledgments	65

8. References	66

8.1. Normative References	66

8.2. Informative References	66

9. Author's Addresses	67

Intellectual Property Statement	68

10. Disclaimer of Validity	68

11. Copyright Statement	69

# Introduction

The SatLabs Group [SATLABS] is an international non-profit EEIG (European Economic Interest Grouping) committed to large-scale adoption and deployment of the Digital Video Broadcasting Return Channel via Satellite (DVB-RCS) standard [ETSI-RCS]. SatLabs members are service providers, satellite operators, system integrators, terminal manufacturers and technology providers with an interest in DVB-RCS.

Since its creation in 2001, the main goal of the SatLabs Group has been to achieve interoperability between DVB-RCS terminals and systems. Therefore, the Group has defined the SatLabs Qualification Program which provides an independent certification process for DVB-RCS Terminals based on System Recommendations defined by SatLabs. To enhance products interoperability, beyond the physical and MAC layers mechanisms defined in the DVB-RCS standard, SatLabs has expanded its Recommendations in the field of DVB-RCS terminal management [SATLABS]. As a part of this effort, SatLabs has specified a common SNMP Management Information Base (MIB) for DVB-RCS terminals, which is defined in this document.

# Conventions used in this document

This memo defines a portion of the Management Information Base (MIB) for use with network management protocols in the Internet community.

For a detailed overview of the documents that describe the current Internet-Standard Management Framework, please refer to section 7 of RFC 3410 [RFC3410].

Managed objects are accessed via a virtual information store, termed the Management Information Base or MIB. MIB objects are generally accessed through the Simple Network Management Protocol (SNMP). Objects in the MIB are defined using the mechanisms defined in the Structure of Management Information (SMI).  This memo specifies a MIB module that is compliant to the SMIv2, which is described in STD 58, RFC 2578 [RFC2578], STD 58, RFC 2579 [RFC2579] and STD 58, RFC 2580 [RFC2580].

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT" "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in RFC 2119. [RFC2119].

## MIB Module

The MIB module in this document uses the following OBJECT IDENTIFIER values assigned by IANA:

+------------+---------------------------+

| Descriptor | OBJECT IDENTIFIER value   |

+------------+---------------------------+

|dvbrcsIfMib |{ mib-2 transmission 239 } |

+------------+---------------------------+

Table 1: Object Identifiers for the MIB

These values have been assigned for this MIB under the 'mib-2.transmission' subtree.

# Definitions

DVBRCS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, transmission, NOTIFICATION-TYPE

FROM SNMPv2-SMI

TEXTUAL-CONVENTION, DisplayString,

PhysAddress, RowStatus    FROM SNMPv2-TC;

dvbrcsMIB MODULE-IDENTITY

LAST-UPDATED "200707061200Z"

ORGANIZATION "The SatLabs Group"

CONTACT-INFO

"The SatLabs Group

Web:	www.satlabs.org

E-mail: info@satlabs.org      "

DESCRIPTION

"DVB-RCS MIB subtree. Draft version B."

REVISION "200707061200Z"

DESCRIPTION

"Draft revision B of this MIB module."

::= { transmission 239 }

===================================================================

-- Textual Conventions

===================================================================

SatLabsOptionMap ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION	"This textual convention enumerates the declaration of the SatLabs defined options.

(0) refers to the most significant bit."

REFERENCE	"SatLabs System Recommendation version 2 available at www.satlabs.org"

SYNTAX BITS 	{

mpegTrf(0),

coarseSync(1),

wideHop(2),

fastHop(3),

dynamicMfTdma(4),

routeId(5),

contentionSync(6),

qpskLow(7),

mod16Apsk(8),

mod32Apsk(9),

normalFec(10),

multiTs(11),

gsTs(12),

enhQoS(13),

pep(14),

http(15),

ftp(16),

dhcp(17),

dns(18),

avbdcRep(19),

chIdStrict(20),

rcstPara(21),

nlid(22),

snmpAll(23),

installLog(24),

fwdLinkStatus(25),

enhClassifier(26),

spare1(27),

spare2(28),

spare3(29),

spare4(30),

spare5(31),

}

===================================================================

-- object type definitions

===================================================================

dvbrcsMIBObjects  	OBJECT IDENTIFIER ::= {dvbrcsMIB 1}

dvbrcsConformance   	OBJECT IDENTIFIER ::= {dvbrcsMIB 2}

rcst    	OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 1}

fwdLink    	OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 2}

rtnLink    	OBJECT IDENTIFIER ::= {dvbrcsMIBObjects 3}

rcstSystem    		OBJECT IDENTIFIER ::= {rcst 1}

rcstNetwork	   	OBJECT IDENTIFIER ::= {rcst 2}

rcstInstall	   	OBJECT IDENTIFIER ::= {rcst 3}

rcstQos    		OBJECT IDENTIFIER ::= {rcst 4}

rcstControl	    	OBJECT IDENTIFIER ::= {rcst 5}

rcstState	    	OBJECT IDENTIFIER ::= {rcst 6}

fwdConfig		OBJECT IDENTIFIER ::= {fwdLink 1}

fwdStatus		OBJECT IDENTIFIER ::= {fwdLink 2}

rtnConfig		OBJECT IDENTIFIER ::= {rtnLink 1}

rtnStatus		OBJECT IDENTIFIER ::= {rtnLink 2}

===================================================================

-- rcstSystem sub-tree object types

===================================================================

systemMibRevision		OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"This object allows the SNMP agent to report the implemented MIB module revision.

The supported REVISION of this module is reported."

::= {rcstSystem 1}

===================================================================

-- options declared according to the textual conventions

===================================================================

systemSatlabsOptions	OBJECT-TYPE

SYNTAX		SatLabsOptionMap

MAX-ACCESS	read-only

STATUS		current

DESCRIPTION

"Indicates the options supported as defined in SatLabs system recommendations version 2. A value of 1 indicates that the respective option is supported. The mapping to the options is to be understood as described here. (0) refers to the most significant bit.

mpegTrf(0) -> MPEG_TRF

coarseSync(1) -> COARSE_SYNC

wideHop(2) -> WIDE_HOPP

fastHop(3) -> FAST_HOPP

dynamicMfTdma(4) -> Dynamic_MF_TDMA

routeId(5) -> ROUTE_ID

contentionSync(6) -> CONTENTION_SYNC

qpskLow(7) -> QPSKLOW

mod16Apsk(8) -> 16APSK

mod32Apsk(9) -> 32APSK

normalFec(10) -> NORMALFEC

multiTs(11) -> MULTITS

gsTs(12) -> GSTS

enhQoS(13) -> ENHQOS

pep(14) -> PEP

http(15) -> HTTP

ftp(16) -> FTP

dhcp(17) -> DHCP

dns(18) -> DNS

avbdcRep(19) -> AVBDC_REP

chIdStrict(20) -> CHID_STRICT

rcstPara(21) -> RCST_PARA

nlid(22) -> NLID

snmpAll(23) -> SNMPALL

installLog(24) -> INSTALL_LOG

fwdLinkStatus(25) -> FWDLINKSTATUS

enhClassifier(26) -> ENHCLASSIFIER

The support of specific options mandates the support of specific objects and access levels."

REFERENCE	"SatLabs System Recommendation version 2 available at www.satlabs.org"

::= {rcstSystem 2}

systemLocation		OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The string shall have 31 characters in the following format:

<xxxx.xxx>,<a>,<yyyyy.yyy>,<b>,<zzzz.z>,M

where x, y and z represents digits,

a=N or S,

b=E or W,

Reading the digits from left to right:

‘x’ – 7 latitude digits;

x digits 1-2 contain the degrees,

x digits 3-7 contain the minutes in decimal;

‘y’ – 8 longitude digits;

y digits 1-3 contain the degrees,

y digits 4-8 contain the minutes in decimal;

‘z’ 5 altitude digits; meters above sea level in decimal;

‘.’ is the decimal point;

‘,’ is the field separator;

‘M’ is the indicator for altitude meters.

This format is a modified subset of the NMEA 0183 format for Global Positioning System Fix Data."

::= {rcstSystem 3}

systemOduAntennaSize	  OBJECT-TYPE

SYNTAX              DisplayString

UNITS		       "cm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This object gives the diameter of the antenna. This value

shall be given in centimeters. It is defined at installation. The

object can be used in conjunction with environmental requirements."

::= {rcstSystem 4}

systemOduAntennaGain	OBJECT-TYPE

SYNTAX              Integer32

UNITS		       "x0.1 dBi"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the antenna peak gain of the ODU and

shall be defined by the installer. The gain shall be given

in tenth of dBi for more flexibility, i.e. 46.5 dBi will be

represented by 465. Defined at installation."

::= {rcstSystem 5}

systemOduSspa	OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 W"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This field describes the SSPA installed in the ODU and

shall be defined by the installer. The power shall be given

in tenth of Watt for more flexibility, i.e. 0,5 W will be

represented by 5, 1 W by 10 and 2 W by 20.

Defined at installation."

::= {rcstSystem 6}

systemOduTxType	OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of transmitter installed in the ODU."

::= {rcstSystem 7}

systemOduRxType	OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes the type of LNB installed in the ODU, with

information such as vendor type, ouput type (single, twin,

quad,…), etc.. "

::= {rcstSystem 8}

systemOduRxBand	OBJECT-TYPE

SYNTAX              INTEGER   {

oduHighRxBand (0),

oduLowRxBand  (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Describes whether High Band or Low Band is selected in

the LNB. Specifying High Band results in activation of a

18-26 kHz tone with 0.4-0.8 Vpp in the Rx IFL cable:

(0)	- High Band

(1)	- Low Band "

::= {rcstSystem 9}

systemOduRxLO 	OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Frequency of LNB Local Oscillator (in 100 Hz)"

::= {rcstSystem 10}

systemOduTxLO		OBJECT-TYPE

SYNTAX              Integer32

UNITS		       "x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency of BUC Local Oscillator (in 100 Hz) "

::= {rcstSystem 11}

===================================================================

-- rcstInstall sub-tree object types

===================================================================

installAntennaAlignmentState  OBJECT-TYPE

SYNTAX             INTEGER {

antennaAlignmentStart	(1),

antennaAlignmentDeny	(2),

antennaAlignmentContinue (3),

antennaAlignmentStop	(4),

antennaAlignmentSuccess	(5),

antennaAlignmentFail	(6)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Indicates the state:

(1)-Start;

(2)-Deny;

(3)-Continue;

(4)-Stop;

(5)-Success;

(6)-Fail"

::= {rcstInstall 1}

installCwFrequency OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x100 Hz"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Frequency at which the carrier is put up (in 100 Hz).

Minimum required precision is 1 kHz."

::= {rcstInstall 2}

installCwMaxDuration OBJECT-TYPE

SYNTAX              Integer32

UNITS		"seconds"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time after which the carrier must be put down (in seconds)"

::= {rcstInstall 3}

installCwPower	OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specified in tenth of dBm. This parameter allows for fine

(manual) tuning of the power setup. Minimum required precision

is 1 dBm. It must be configurable while CW signal is on. Once

the power has been properly adjusted, the power setup is stored

in rtnConfigDefIfLevel."

::= {rcstInstall 4}

installCoPolReading OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Pol measured value (in 0.1 dB)"

::= {rcstInstall 5}

installXPolReading	OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Pol measured value (in 0.1 dB)"

::= {rcstInstall 6}

installCoPolTarget OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Co-Pol target (in 0.1 dB) "

::= {rcstInstall 7}

installXPolTarget		OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Cross-Pol target (in 0.1 dB)"

::= {rcstInstall 8}

installStandByDuration	OBJECT-TYPE

SYNTAX              Integer32

UNITS		"seconds"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time to wait in stand-by mode (in seconds)"

::= {rcstInstall 9}

installTargetEsNo	OBJECT-TYPE

SYNTAX              Integer32(0..315)

UNITS		"x0.1 dB"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This value describes the wanted Es/N0 value that enables

operation of the return link with the required error

performance. The values shall be given in tenth of dB and

the initial value shall be equal to 7 dB. The range shall be

from 0 dBm to 31.5 dBm with a precision of 0.1 dB."

DEFVAL		{ 70 }

::= {rcstInstall 10}

===================================================================

-- rcstNetwork sub-tree object types

===================================================================

networkOamIpAddr          OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"OAM IP Address of the RCST. This object used with both ip and

interfaces MIB-II subgroups determines uniquely the interface

through which OAM traffic is passing through.

Note that the OAM IP address may be statically or dynamically

assigned. It is system dependent whether the OAM IP address

and the Traffic IP address are the same address."

::= {rcstNetwork 1}

networkOamIpNetworkMask      OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Network Mask for the OAM IP Address."

::= {rcstNetwork 2}

networkOamIpAddrAssign     OBJECT-TYPE

SYNTAX             INTEGER	{

oamIpAddressStatic	(1),

oamIpAddressDynamic	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Identifies whether the OAM IP address is statically

('static' - 1) or dynamically ('dynamic' - 2) assigned."

::= {rcstNetwork 3}

networkLanIpAddress        OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the LAN interface"

::= {rcstNetwork 4}

networkLanIpMask           OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Mask for the LAN interface"

::= {rcstNetwork 5}

networkAirInterfaceDefaultGateway   OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Default Gateway for the air interface"

::= {rcstNetwork 6}

networkDnsServerIpAddress    OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the DNS server in the NCC."

::= {rcstNetwork 7}

networkNccMgtIpAddress       OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"IP address of the management server in the NCC."

::= {rcstNetwork 8}

networkConfigFileDownloadUrl  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST."

::= {rcstNetwork 9}

networkInstallLogFileDownloadUrl    OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file to download.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname. Hostname can

only be used if DNS is supported by the RCST. The installation

log file can be created on the installer's computer

and downloaded to the RCST."

::= {rcstNetwork 10}

networkConfigFileUploadUrl   OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path name for the configuration file upload.

It includes the protocol type (tftp or ftp) and the

associated server IP address or hostname.Hostname can

only be used if DNS is supported by the RCST."

::= {rcstNetwork 11}

networkLogFileUploadUrl        OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the event log file. It includes the protocol

type (tftp or ftp) and the associated server IP address

or hostname. Hostname can only be used if DNS is supported

by the RCST."

::= {rcstNetwork 12}

networkInstallLogFileUploadUrl  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Full path of the installation log file. It includes the

protocol type (tftp or ftp) and the associated server

IP address or hostname. Hostname can only be used if DNS is

supported by the RCST. The installation log file can be

retrieved from the RCST by the NCC or by the installer

via the LAN."

::= {rcstNetwork 13}

===================================================================

--  rtnConfig sub-tree object types

===================================================================

rtnconfigMaxEirp            OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Maximum allowed EIRP in tenth of dBm on the return link."

::= {rtnConfig 1}

rtnconfigDefIfLevel        OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dBm"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Default transmitted IF power level, specified in tenth of dBm,

out of the IDU for sending a CSC burst at RCST reboot or

power on. Applies to all signals transmitted by RCST

(CW and traffic) immediately."

::= {rtnConfig 2}

===================================================================

-- rtnStatus sub-tree object types

===================================================================

rtnstatusEbN0        OBJECT-TYPE

SYNTAX              Integer32

UNITS		"x0.1 dB"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The EbN0 value reported for the return link, referenced to the regular SYNC burst transmission."

::= {rtnStatus 1}

rtnstatusSFduration        OBJECT-TYPE

SYNTAX              Integer32 (250..7500)

UNITS		"0.1 ms"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The duration of the currently applied return link super-frame structure, in tenths of milliseconds."

::= {rtnStatus 2}

rtnstatusPayloadUnit        OBJECT-TYPE

SYNTAX              INTEGER   {

unitATM    (0),

unitMPEG   (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates if the payload unit used for the return link is ATM or MPEG."

::= {rtnStatus 3}

===================================================================

-- fwdConfig sub-tree object types

===================================================================

fwdStartTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF FwdStartEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"Lists Forward Links attachment points (e.g. different for installation and operation). The table describes the forward link parameters used for the start-up stream with the NCC."

::={fwdConfig 1}

fwdStartEntry        OBJECT-TYPE

SYNTAX            FwdStartEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link StartConfig table."

INDEX   { fwdStartIndex, fwdStartPopId }

::= { fwdStartTable 1 }

FwdStartEntry ::= SEQUENCE {

fwdStartIndex		Integer32,

fwdStartPopId		Integer32,

fwdStartFrequency 	Integer32,

fwdStartPolar     	INTEGER,

fwdStartFormat  	INTEGER,

fwdStartRolloff   	INTEGER,

fwdStartSymbolRate  	Integer32,

fwdStartInnerFec    	INTEGER,

fwdStartRowStatus   	RowStatus

}

fwdStartIndex        OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Table index."

::={fwdStartEntry 1}

fwdStartPopId        OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Population identifier associated with the start-up

forward link:

-1: any (auto)

0-65535: specific StartPopId

If 'any' is set, the RCST will assume membership of any annonced population ID and will

commence with logon in accordance with this assumption."

::={fwdStartEntry 2}

fwdStartFrequency        OBJECT-TYPE

SYNTAX              Integer32

UNITS		"100 kHz"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Frequency of the start transponder carrying a

Network Information Table to which any RCST shall

trigger to acquire forward link. ts value shall be given

in multiple of 100 kHz."

::={fwdStartEntry 3}

fwdStartPolar  OBJECT-TYPE

SYNTAX              INTEGER   {

linearHorizontal (0),

linearVertical   (1),

circularLeft     (2),

circularRight    (3)

}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the start transponder

carrying an Network Information Table to which any RCST shall

trigger to acquire forward link:

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={fwdStartEntry 4}

fwdStartFormat    OBJECT-TYPE

SYNTAX              INTEGER {

auto 	(-1),

dvbs 	(0),

dvbs2 	(1)

}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the transmission format standard applied for the start

transport stream. The start transport stream carries a

Network Information Table that the RCST uses for acquiring

the forward link signaling. Supported values are:

-1: unspecified     (automatic format acquisition is assumed)

0: DVB-S            (mandatory if DVBS support is claimed)

1: DVB-S2 CCM 	    (mandatory if CCM support is claimed)

2: DVB-S2 ACM 	    (mandatory if ACM support is claimed)

This allows the RCST to discriminate between CCM and ACM when selecting FL.

The support of automatic format selection is optional. One or several of the other

format selections must be supported, according to the claimed SatLabs profile support."

::={fwdStartEntry 5}

fwdStartRolloff    OBJECT-TYPE

SYNTAX              INTEGER {

autoRolloff 	(0),

rolloff020 	(1),

rolloff025 	(2),

rolloff035	(3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Specifies the roll-off applied on the start transponder.

The start transponder carries a Network Information Table

that the RCST uses for acquiring the forward link signaling.

Supported values are:

0: any (auto)

1: 0.20

2: 0.25

3: 0.35

4-n: reserved"

::={fwdStartEntry 6}

fwdStartSymbolRate    OBJECT-TYPE

SYNTAX              Integer32

UNITS		"100 symbols/s"

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the symbol rate on the start transponder

carrying a Network Information Table to which any RCST

shall trigger to acquire forward link. Its value shall be

given in multiple of 100 symbols/s."

::={fwdStartEntry 7}

fwdStartInnerFec    OBJECT-TYPE

SYNTAX              INTEGER     {

autoFec		(-1),

fecRate1/2	(0),

fecRate2/3	(1),

fecRate3/4	(2),

fecRate5/6	(3),

fecRate7/8	(4),

fecRate8/9	(5),

fecRate3/5	(6),

fecRate4/5	(7),

fecRate9/10	(8),

fecRate2/5	(12),

fecRate1/3	(13),

fecRate1/4	(14),

noInnerCode	(15)				}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction used on the

start transponder carrying a NIT to which any RCST shall

trigger to acquire forward link.

Supported values are:

autoFec		(-1),

fecRate1/2	(0),

fecRate2/3	(1),

fecRate3/4	(2),

fecRate5/6	(3),

fecRate7/8	(4),

fecRate8/9	(5),

fecRate3/5	(6),

fecRate4/5	(7),

fecRate9/10	(8),

fecRate2/5	(12),

fecRate1/3	(13),

fecRate1/4	(14),

noInnerCode	(15)

The support of autoFec is optional."

::={fwdStartEntry 8}

fwdStartRowStatus    OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL  { active }

::={fwdStartEntry 9}

===================================================================

--  fwdStatus sub-tree object types

===================================================================

fwdStatusPopId		OBJECT-TYPE

SYNTAX              Integer32 (0..65535)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Population identifier applied at log-on:

0-65535: specific StartPopId

If the RCST was allowed to logon any population, the RCST will report the base number of the announced

population ID indicated	by the RMT linkage descriptor used at logon."

::={fwdStatus 1}

fwdStatusTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF FwdStatusEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the current status of Forward Link interfaces."

::={fwdStatus 2}

fwdStatusEntry        OBJECT-TYPE

SYNTAX            FwdStatusEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Forward Link Status table. Each entry is associated with a physical interface. A RCST SHALL support at least one entry."

INDEX   { fwdStatusIndex, fwdStatusIfReference }

::= { fwdStatusTable 1 }

FwdStatusEntry ::= SEQUENCE {

fwdStatusIndex			Integer32,

fwdStatusIfReference	Integer32,

fwdStatusNetId       	Integer32,

fwdStatusNetName       	DisplayString,

fwdStatusFormat	  	INTEGER,

fwdStatusFrequency    	Integer32,

fwdStatusPolar  		INTEGER,

fwdStatusInnerFec 		INTEGER,

fwdStatusSymbolRate   	Integer32,

fwdStatusRolloff    	INTEGER,

fwdStatusModulation  	INTEGER,

fwdStatusFecFrame     	INTEGER,

fwdStatusPilot			INTEGER,

fwdStatusBer 			Integer32,

fwdStatusCnr      		Integer32,

fwdStatusRxPower      	Integer32,

fwdStatusRowStatus		RowStatus

}

fwdStatusIndex    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Table index."

::={fwdStatusEntry 1}

fwdStatusIfReference    OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Cross reference to the interface table"

::={fwdStatusEntry 2}

fwdStatusNetId      OBJECT-TYPE

SYNTAX              Integer32

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Interactive network identifier of the forward

link (from RMT)"

::={fwdStatusEntry 3}

fwdStatusNetName      OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"The name of the interactive network of the forward

link (from RMT)"

::={fwdStatusEntry 4}

fwdStatusFormat  OBJECT-TYPE

SYNTAX              INTEGER  {

dvbs 		(0),

dvbs2ccm 	(1),

dvbs2acm 	(2),

reservedFormat  (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Specifies the transmission format applied on the forward link. Supported values are (from RMT):

0: DVB-S

1: DVB-S2 using CCM

2: DVB-S2 using ACM

3: reserved"

::={fwdStatusEntry 5}

fwdStatusFrequency    OBJECT-TYPE

SYNTAX           	Integer32

UNITS			"100 kHz"

MAX-ACCESS    	read-only

STATUS           	current

DESCRIPTION

"An estimate of the frequency of the forward link. Its value shall

be given in multiple of 100 kHz."

::={fwdStatusEntry 6}

fwdStatusPolar    OBJECT-TYPE

SYNTAX            INTEGER   {

linearHorizontal (0),

linearVertical   (1),

circularLeft     (2),

circularRight    (3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"2-bit field giving the polarization of the forward link

(from RMT):

00: linear and horizontal

01: linear and vertical

10: circular left

11: circular right"

::={fwdStatusEntry 7}

fwdStatusInnerFec    OBJECT-TYPE

SYNTAX              INTEGER     {

unknown		(-1),

fecRate1/2	(0),

fecRate2/3	(1),

fecRate3/4	(2),

fecRate5/6	(3),

fecRate7/8	(4),

fecRate8/9	(5),

fecRate3/5	(6),

fecRate4/5	(7),

fecRate9/10	(8),

fecRate2/5	(12),

fecRate1/3	(13),

fecRate1/4	(14),

noInnerCode	(15)				}

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Specifies the inner Forward Error Correction used on the

forward link for transmission to the RCST.

Supported values are:

unknown		(-1),

fecRate1/2	(0),

fecRate2/3	(1),

fecRate3/4	(2),

fecRate5/6	(3),

fecRate7/8	(4),

fecRate8/9	(5),

fecRate3/5	(6),

fecRate4/5	(7),

fecRate9/10	(8),

fecRate2/5	(12),

fecRate1/3	(13),

fecRate1/4	(14),

noInnerCode	(15)

The RCST will report a value that has been used for transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unkown'."

::={fwdStatusEntry 8}

fwdStatusSymbolRate    OBJECT-TYPE

SYNTAX              Integer32

UNITS		"100 symbols/s"

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"An estimate of the symbol rate of the forward link.

Its value shall be given in multiple of 100 symbol/s."

::={fwdStatusEntry 9}

fwdStatusRolloff    OBJECT-TYPE

SYNTAX              INTEGER   {

undefRolloff	(0),

r020 		(1),

r025 		(2),

r035    	(3)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"An estimate of the roll-off applied on the forward link.

Supported values are:

0: undefined

1: 0.20

2: 0.25

3: 0.35

4-15: reserved"

::={fwdStatusEntry 10}

fwdStatusModulation    OBJECT-TYPE

SYNTAX              INTEGER   {

unknown		(0),

mBPSK 		(1),

mQPSK 		(2),

m8PSK    	     (3),

m16APSK		(4),

m32APSK		(5)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the modulation on the forward link used for transmission to the RCST.

0: unknown

1: BPSK

2: QPSK

3: 8PSK

4: 16APSK

5: 32APSK

The RCST will report a value that has been used for transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unkown'."

::={fwdStatusEntry 11}

fwdStatusFecFrame    OBJECT-TYPE

SYNTAX              INTEGER {

unknown	(0),

shortframe	(1),

longframe	(2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the frame length used on the forward link for transmission to the RCST.

Supported values are:

0: Unknown

1: Normal frame

2: Short frame

The RCST will report a value that has been used for transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unkown'."

::={fwdStatusEntry 12}

fwdStatusPilot		OBJECT-TYPE

SYNTAX              INTEGER {

unknown	(0),

pilotNotused	(1),

pilotUsed	(2)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates whether pilots are used on the forward link for transmission to the RCST.

Supported values are:

0: Unknown

1: Pilots are not used

2: Pilot not used

The RCST will report a value that has been used for transmission to the RCST within the most recent 60 seconds.

If this is not relevant, the RCST will report 'unkown'."

::={fwdStatusEntry 13}

fwdStatusBer		OBJECT-TYPE

SYNTAX       Integer32

UNITS		"exponent of 10"

MAX-ACCESS	read-only

STATUS    	current

DESCRIPTION

"Provides the RCST BER on the Forward Link in log10 units."

::={fwdStatusEntry 14}

fwdStatusCnr		OBJECT-TYPE

SYNTAX    	Integer32

UNITS		"0.1 dB"

MAX-ACCESS  	read-only

STATUS       current

DESCRIPTION

"Provides the RCST CNR on the Forward Link in 0.1 dB units."

::={fwdStatusEntry 15}

fwdStatusRxPower	OBJECT-TYPE

SYNTAX      	Integer32

UNITS		"0.1 dBm"

MAX-ACCESS	read-only

STATUS 		current

DESCRIPTION

"Provides the power level of the Forward Link as received at the IDU, in 0.1 dBm units."

DEFVAL { -500 }

::={fwdStatusEntry 16}

fwdStatusRowStatus    OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={fwdStatusEntry 17}

===================================================================

-- rcstQos sub-tree object types

===================================================================

pktClassTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF PktClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table describes the packet classification used in the DVB-RCS terminal. The number of entries is specified by PktClassIndex. "

::={rcstQos 1}

pktClassEntry        OBJECT-TYPE

SYNTAX            PktClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the packet classification table. One object type of each entry may have a value in the active range

(a non-default value).The other object types are then assumed set to 'inactive'. The entry with the lowest index

value takes precedence when classifying a packet."

INDEX   { pktClassIndex }

::= { pktClassTable 1 }

PktClassEntry ::= SEQUENCE {

pktClassIndex			INTEGER,

pktClassDscpLow         	Integer32,

pktClassDscpHigh        	Integer32,

pktClassDscpMarkValue   	Integer32,

pktClassIPProtocol      	Integer32,

pktClassIPSrcAddr       	IpAddress,

pktClassIPSrcAddrMask   	IpAddress,

pktClassIPDstAddr       	IpAddress,

pktClassIPDstAddrMask   	IpAddress,

pktClassSrcPortLow      	Integer32,

pktClassSrcPortHigh     	Integer32,

pktClassDstPortLow      	Integer32,

pktClassDstPortHigh     	Integer32,

pktClassVlanUserPri		Integer32,

pktClassPhbAssociation  	Integer32,

pktClassRowStatus		RowStatus

}

pktClassIndex    OBJECT-TYPE

SYNTAX              INTEGER

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index automatically incremented by one at row creation."

::={pktClassEntry 1}

pktClassDscpLow OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low value of a range

of DSCP values to which a packet is compared. A value of 0 is used to inactivate."

DEFVAL		{ 0 }

::={pktClassEntry 2}

pktClassDscpHigh OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high value of a range of

DSCP values to which a packet is compared. A value of 63 is used to inactivate."

DEFVAL		{ 63 }

::={pktClassEntry 3}

pktClassDscpMarkValue OBJECT-TYPE

SYNTAX              Integer32 (-1..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object is the DSCP value used to mark the packet,

64 indicates no DSCP marking, possible DSCP marks values

are (0..63)"

DEFVAL		{ -1 }

::={pktClassEntry 4}

pktClassIPProtocol OBJECT-TYPE

SYNTAX              Integer32 (-1..255)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP protocol to which a

packet is compared. A value of 255 indicates inactive."

DEFVAL		{ -1 }

::={pktClassEntry 5}

pktClassIPSrcAddr     OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP source address to which a

packet is compared."

::={pktClassEntry 6}

pktClassIPSrcAddrMask  OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP source

address will be matched. A mask of all zeros inidcates that the selectivity is inactive."

DEFVAL		{ 0 }

::={pktClassEntry 7}

pktClassIPDstAddr      OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the IP destination address to which a

packet is compared."

::={pktClassEntry 8}

pktClassIPDstAddrMask  OBJECT-TYPE

SYNTAX              IpAddress

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies which bits of the IP destination

address will be matched. A mask of all zeros indicates that the selectivity is inactive."

DEFVAL		{ 0 }

::={pktClassEntry 9}

pktClassSrcPortLow OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low range of the source

port to which a packet is compared. A value of -1 indicates that the selectivity is inactive."

DEFVAL		{ -1 }

::={pktClassEntry 10}

pktClassSrcPortHigh  OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high range of the source

port to which a packet is compared. A value of -1 indicates that the selectivity is inactive."

DEFVAL		{ -1 }

::={pktClassEntry 11}

pktClassDstPortLow   OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the low range of the destination

port to which a packet is compared. A value of -1 indicates that the selectivity is inactive."

DEFVAL		{ -1 }

::={pktClassEntry 12}

pktClassDstPortHigh  OBJECT-TYPE

SYNTAX              Integer32 (-1..65535)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the high range of the destination

port to which a packet is compared. A value of -1 indicates that the selectivity is inactive."

DEFVAL		{ -1 }

::={pktClassEntry 13}

pktClassVlanUserPri OBJECT-TYPE

SYNTAX              Integer32 (-1..7)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object specifies the VLAN User Priority

to which a packet is compared. A value of -1 indicates that the selectivity is inactive."

DEFVAL		{ -1 }

::={pktClassEntry 14}

pktClassPhbAssociation OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Associate the filter entry to a specific PHB (refer to phbIdentifier)."

::={pktClassEntry 15}

pktClassRowStatus   OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

::={pktClassEntry 16}

===================================================================

-- PhbMappingTable

===================================================================

phbMappingTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF PhbMappingEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of PHB MIB entries. It describes the PHB mapping to the Request Class."

::={rcstQos 2}

phbMappingEntry        OBJECT-TYPE

SYNTAX            PhbMappingEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the PHB mapping table."

INDEX   { phbIdentifier }

::= { phbMappingTable 1 }

PhbMappingEntry ::= SEQUENCE {

phbIdentifier       		Integer32,

phbName             		DisplayString,

phbRequestClassAssociation  	Integer32,

phbMappingRowStatus         	RowStatus

}

phbIdentifier OBJECT-TYPE

SYNTAX              Integer32 (0..63)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Identification of the PHB (0..63)."

::={phbMappingEntry 1}

phbName  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"The name of the PHB."

::={phbMappingEntry 2}

phbRequestClassAssociation  OBJECT-TYPE

SYNTAX               Integer32 (1..16)

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"This object is an association of this PHB

to a Request class (by reference to a Request Class index)."

::={phbMappingEntry 3}

phbMappingRowStatus   OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-create

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={phbMappingEntry 4}

===================================================================

-- RequestClassTable

===================================================================

requestClassTable      OBJECT-TYPE

SYNTAX              SEQUENCE OF RequestClassEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table is a list of Request class entries. This class describes the layer 2 QoS objects."

::={rcstQos 3}

requestClassEntry        OBJECT-TYPE

SYNTAX            RequestClassEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the Request class table."

INDEX   { requestClassIndex }

::= { requestClassTable 1 }

RequestClassEntry ::= SEQUENCE {

requestClassIndex		Integer32,

requestClassName			DisplayString,

requestClassChanId		Integer32,

requestClassVccVpi      	Integer32,

requestClassVccVci      	Integer32,

requestClassPidPoolReference	Integer32,

requestClassCra             	INTEGER,

requestClassRbdcMax         	INTEGER,

requestClassRbdcTimeout     	INTEGER,

requestClassVbdcMax         	INTEGER,

requestClassVbdcTimeout     	INTEGER,

requestClassVbdcMaxBackLog  	INTEGER,

requestClassAvbdcRepTime    	INTEGER,

requestClassRowStatus       	RowStatus

}

requestClassIndex    OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the Request Class table. A total of 16 entries is supported."

::={requestClassEntry 1}

requestClassName   OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Name of the Request class."

::={requestClassEntry 2}

requestClassChanId  OBJECT-TYPE

SYNTAX              Integer32 (0..15)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Channel id of the Request class."

::={requestClassEntry 3}

requestClassVccVpi OBJECT-TYPE

SYNTAX              Integer32 (0..255)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Defines VPI used for the Request class (ATM profile)."

::={requestClassEntry 4}

requestClassVccVci  OBJECT-TYPE

SYNTAX              Integer32 (0..65535)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Defines VCI used for the Request class (ATM profile)."

::={requestClassEntry 5}

requestClassPidPoolReference OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Reference to the PID pool applicable for the request class."

::={requestClassEntry 6}

requestClassCra  OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "bits/s"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Define CRA level for the Request class in bit per second (b/s)."

::={requestClassEntry 7}

requestClassRbdcMax OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "2 kbits/s"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

" Max RBDC that can be requested for the Request class, in number of 2 Kb/s"

::={requestClassEntry 8}

requestClassRbdcTimeout OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "superframes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Persistence of the RBDC request, expressed in superframes"

::={requestClassEntry 9}

requestClassVbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "ATM cells/MPEG packets"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Max VBDC that can be requested for the Request class,

in payload units (one ATM cell or one MPEG packet) per superframe"

::={requestClassEntry 10}

requestClassVbdcTimeout OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "superframes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Time after which the RCST considers that the pending requests are lost.

The RCST may issue new request for that traffic. VBDC Timeout is

expressed in superframes."

::={requestClassEntry 11}

requestClassVbdcMaxBackLog OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "bytes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"VBDC back Log per Request class. (expressed in bytes)"

::={requestClassEntry 12}

requestClassAvbdcRepTime OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "superframes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Optional parameter. AVBDC requests can be sent periodically,

according to the value of that parameter.

AVBDCRepTime is expressed in superframes."

::={requestClassEntry 13}

requestClassRowStatus  OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={requestClassEntry 14}

===================================================================

-- the table of PID pools

===================================================================

pidPoolTable OBJECT-TYPE

SYNTAX              SEQUENCE OF PidPoolEntry

MAX-ACCESS          not-accessible

STATUS              current

DESCRIPTION

"This table contains the PID pools. A PID value may occur in more than one PID pool. Each PID value can effectively

occur only once in each pool.  Only one Pid per PID pool is required for short term QoS support."

::={rcstQos 4}

pidPoolEntry        OBJECT-TYPE

SYNTAX            PidPoolEntry

MAX-ACCESS        not-accessible

STATUS            current

DESCRIPTION

"An entry in the PID list table."

INDEX   { pidPoolIndex, pidIndex }

::= { pidPoolTable 1 }

PidPoolEntry ::= SEQUENCE {

pidPoolIndex		Integer32,

pidIndex		Integer32,

pidValue		Integer32,

pidPoolRowStatus	RowStatus

}

pidPoolIndex OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the Pid pool in the PID pool table."

::={pidPoolEntry 1}

pidIndex OBJECT-TYPE

SYNTAX              Integer32 (1..16)

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Index of the PID entry within the PID pool."

::={pidPoolEntry 2}

pidValue OBJECT-TYPE

SYNTAX              Integer32 (0..8191)

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Define one of the PIDs to be used in a PID pool of pidPoolIndex."

::={pidPoolEntry 3}

pidPoolRowStatus  OBJECT-TYPE

SYNTAX              RowStatus

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Standard SNMP row status"

DEFVAL { active }

::={pidPoolEntry 4}

qosGlobalRbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "2 kbits/s"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global RBDC max is for information only and it is optional.

(expressed in number of 2 kb/s)"

::={rcstQos 5}

qosGlobalVbdcMax  OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "ATM cells/MPEG packets"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global VBDC max is for information only and it is optional.(expressed in payload units/superframe)"

::={rcstQos 6}

qosGlobalVbdcMaxBackLog OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "bytes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global VBDC back log is used only if the VBDC back log is not

configured in the Request class.	(expressed in bytes)"

::={rcstQos 7}

qosGlobalAvbdcRepTime  OBJECT-TYPE

SYNTAX              INTEGER

UNITS			  "superframes"

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"The global AVBDC rep time is optional (expressed in superframes)."

::={rcstQos 8}

qosChannelIdStrictDispatching OBJECT-TYPE

SYNTAX              INTEGER {

not_strict (0),

strict (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Indicates whether the RCST will strictly follow RC association when

signaled through Channel_ID in the TBTP:

No strict association (0)

Strict association (1)"

::={rcstQos 9}

===================================================================

-- rcstState sub-tree object types

===================================================================

rcstMode  OBJECT-TYPE

SYNTAX              INTEGER {

installation (0),

operational (1)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Allows to identify in which mode the RCST is and to return it to the installation mode when needed. Values for the RCST mode are:

Installation (0)

Operational (1)"

::={rcstState 1}

rcstFaultStatus  OBJECT-TYPE

SYNTAX              INTEGER {

nofault (0),

fault (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provide the fault status of the terminal. The fault status management is vendor specific. Values for the Fault Status are:

no fault (0)

fault (1)"

::={rcstState 2}

rcstFwdLinkStatus OBJECT-TYPE

SYNTAX              INTEGER {

notAcquired (0),

acquired 	(1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Forward Link. Values for the

Forward Link Status are:

Not acquired (0)

Acquired (1)"

::={rcstState 3}

rcstRtnLinkStatus   OBJECT-TYPE

SYNTAX              INTEGER {

loggedOff (0),

loggedOn  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Provides the status of the RCST Return Link. Values for the

Return Link Status are:

Logged-off (0)

Logged-on (1)"

::={rcstState 4}

rcstLogUpdated OBJECT-TYPE

SYNTAX              INTEGER {

noUpdate 		(0),

logfileUpdated  (1)

}

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Indicates the existence of an updated log file:

No update (0)

Log file updated (1)

The RCST should remove the Log file updated indication

as the log file is fetched by the NCC."

::={rcstState 5}

rcstCurrentSoftwareVersion  OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Current RCST software version."

::={rcstState 6}

rcstAlternateSoftwareVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Alternate (backup/new) RCST software version."

::={rcstState 7}

rcstActivatedConfigFileVersion OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the most recently activated configuration file. Version is vendor specific."

::={rcstState 8}

rcstDownloadedConfigFileVersion

OBJECT-TYPE

SYNTAX              DisplayString

MAX-ACCESS          read-only

STATUS              current

DESCRIPTION

"Version of the most recently downloaded configuration file. Version is vendor specific. If the

value is different from rcstActivatedConfigFileVersion, it is pending for activation."

::={rcstState 9}

===================================================================

-- rcstControl sub-tree object types

====================================================================

ctrlRebootCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

normal 	(2),

alternate (3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to reboot

(1)- idle

(2)- normal reboot (from current software load)

(3)- reboot from alternate load (swap to alternate load before reboot)"

DEFVAL {1}

::={rcstControl 1}

ctrlRcstTxDisable	OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

disable 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall force the RCST to stop transmission (transmit disabled as defined in [1]):

(1)- idle

(2)- initiate Tx Disabled"

DEFVAL {1}

::={rcstControl 2}

ctrlUserTrafficDisable	OBJECT-TYPE

SYNTAX           INTEGER {

idle		(1),

disable 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall disable user traffic (only RCST management traffic can be transmitted)

(1)- idle

(2)- disable user traffic"

DEFVAL {1}

::={rcstControl 3}

ctrlCwEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable will force the RCST to start transmission of CW, if the RCST is first set to the installation state, and is properly configured for CW transmission:

(1)- off

(2)- on"

DEFVAL {1}

::={rcstControl 4}

ctrlOduTxReferenceEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of 10 MHz reference

clock in the Tx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 5}

ctrlOduTxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Tx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 6}

ctrlOduRxDCEnable OBJECT-TYPE

SYNTAX           INTEGER {

off	(1),

on 	(2),

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Enables activation and deactivation of DC in the Rx IFL cable:

(1) off

(2) on"

DEFVAL {2}

::={rcstControl 7}

ctrlDownloadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle			 (1),

config	 	 (2),

installationLog (3)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST configuration file download process

(1)	idle

(2)	download RCST configuration file from TFTP/FTP server

(3)	download RCST installation log file from TFTP/FTP server

(INSTALL_LOG OPTION)"

DEFVAL {1}

::={rcstControl 8}

ctrlUploadFileCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle			(1),

config			(2),

eventAlarm		(3),

installationLog	(4)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST upload process

(1)	idle

(2)	upload RCST configuration file to TFTP/FTP server

(3)	upload RCST event/alarm log file to TFTP/FTP server

(4)	upload RCST installation log file to TFTP/FTP server

(INSTALL_LOG OPTION)"

DEFVAL {1}

::={rcstControl 9}

ctrlActivateConfigFileCommand  OBJECT-TYPE

SYNTAX           INTEGER {

idle			(1),

activate		(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"Triggers the RCST to use the configuration file and update

its parameters accordingly.  Some RCST implementation may require

a reboot for the parameters to take effect (vendor specific).

(1)	Idle

(2)	activate"

DEFVAL {1}

::={rcstControl 10}

ctrlRcstLogonCommand	OBJECT-TYPE

SYNTAX           INTEGER {

idle	(1),

logon	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST logon

(1) idle

(2) initiate RCST logon"

DEFVAL {1}

::={rcstControl 11}

ctrlRcstLogoffCommand OBJECT-TYPE

SYNTAX           INTEGER {

idle	(1),

logoff	(2)

}

MAX-ACCESS          read-write

STATUS              current

DESCRIPTION

"This variable shall initiate a RCST logoff

(1) idle

(2) initiate RCST logoff"

DEFVAL {1}

::={rcstControl 12}

===================================================================

-- conformance information

====================================================================

rcstGroups    		OBJECT IDENTIFIER ::= {dvbrcsConformance 1}

rcstCompliances    	OBJECT IDENTIFIER ::= {dvbrcsConformance 2}

===================================================================

-- conformance statements

===================================================================

rcstCompliance1 MODULE-COMPLIANCE

STATUS      	current

DESCRIPTION	"The compliance statement for DVB-RCS terminals that are compliant with SatLabs system

recommendations version 2."

MODULE  -- this module

MANDATORY-GROUPS	{rcstSystemGroup, rcstInstallGroup, rcstNetworkGroup, rtnConfigGroup, fwdConfigGroup, fwdStatusGroup, rcstQosGroup, rcstStateGroup, rcstControlGroup}

GROUP       rcstExtSystemGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the SNMPALL option."

GROUP       rcstExtNetworkGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the SNMPALL option."

GROUP       rcstExtInstallGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the INSTALL_LOG option."

GROUP       rcstDnsGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the DNS option."

GROUP       rtnExtConfigGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the SNMPALL option."

GROUP       fwdExtStatusGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the FWDLINKSTATUS option."

GROUP       rcstEnhancedClassifierGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the ENHCLASSIFIER option."

GROUP       rcstMpegQosGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the MPEG option."

GROUP       rcstAvbdcRepQosGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the AVBDC_REP option."

GROUP       rcstGlobalQosGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the RCST_PARA option."

GROUP       rcstGlobalAvbdcRepQosGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support both the RCST_PARA option and the

AVBDC_REP option."

GROUP       rcstStrictQosGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the CHID_STRICT option."

GROUP       rtnStatusGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the SNMPALL option."

GROUP       rcstExtControlGroup

DESCRIPTION "This group is mandatory for an RCST that claims to support the SNMPALL option."

OBJECT systemOduTxType

MIN-ACCESS read-only

DESCRIPTION	"Write access required if the RCST doesn't provide the information."

OBJECT systemOduRxType

MIN-ACCESS read-only

DESCRIPTION	"Write access required if the RCST doesn't provide the information."

OBJECT networkLanIpAddress

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT networkLanIpMask

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT pktClassDscpLow

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if ENHCLASSIFIER support is claimed."

OBJECT pktClassDscpHigh

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if ENHCLASSIFIER support is claimed."

OBJECT pktClassDscpMarkValue

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if ENHCLASSIFIER support is claimed."

OBJECT phbIdentifier

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if SNMPALL support is claimed."

OBJECT phbName

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if SNMPALL support is claimed."

OBJECT phbRequestClassAssociation

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if SNMPALL support is claimed."

OBJECT phbMappingRowStatus

MIN-ACCESS read-only

DESCRIPTION	"Create access only required if SNMPALL support is claimed."

OBJECT requestClassName

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassChanId

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassVccVpi

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassVccVci

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassPidPoolReference

MIN-ACCESS not-accessible

DESCRIPTION	"Read-only access required if MPEG option is claimed. Write access

only required if also SNMPALL support is claimed."

OBJECT requestClassCra

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassRbdcMax

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassRbdcTimeout

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassVbdcMax

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassVbdcTimeout

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassVbdcMaxBacklog

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT requestClassRowStatus

MIN-ACCESS read-only

DESCRIPTION	"Write access only required if SNMPALL support is claimed."

OBJECT pidPoolIndex

MIN-ACCESS not-accessible

DESCRIPTION	"Read-only access required if MPEG option is claimed."

OBJECT pidIndex

MIN-ACCESS not-accessible

DESCRIPTION	"Read-only access required if MPEG option is claimed."

OBJECT pidValue

MIN-ACCESS not-accessible

DESCRIPTION	"Read-only access required if MPEG option is claimed. Write access

only required if also SNMPALL support is claimed."

OBJECT pidPoolRowStatus

MIN-ACCESS not-accessible

DESCRIPTION	"Read-only access required if MPEG option is claimed. Write access

only required if also SNMPALL support is claimed."

::= { rcstCompliances 1 }

===================================================================

-- units of conformance

====================================================================

===================================================================

-- object groups for rcst system

===================================================================

rcstSystemGroup OBJECT-GROUP

OBJECTS {

systemMibVersion,

systemLocation,

systemOduTxType,

systemOduRxType,

systemSatlabsOptions

}

STATUS	current

DESCRIPTION	"A collection of objects providing information applicable for basic device management support."

::= { rcstGroups 1 }

rcstExtSystemGroup OBJECT-GROUP

OBJECTS {

systemOduAntennaSize,

systemOduAntennaGain,

systemOduSspa,

systemOduRxBand,

systemOduRxLO,

systemOduTxLO,

installCwPower

}

STATUS	current

DESCRIPTION	"A collection of objects providing information applicable for extended device management support."

::= { rcstGroups 2 }

===================================================================

-- object groups for rcst installation

===================================================================

rcstInstallGroup OBJECT-GROUP

OBJECTS {

installAntennaAlignmentState,

installCwFrequency,

installCwMaxDuration,

installCoPolReading,

installXPolReading,

installCoPolTarget,

installXPolTarget,

installStandByDuration,

installTargetEsNo,

}

STATUS	current

DESCRIPTION	"A collection of objects providing information applicable for basic installation support."

::= { rcstGroups 3 }

rcstExtInstallGroup OBJECT-GROUP

OBJECTS {

networkInstallLogFileDownloadUrl,

networkInstallLogFileUploadUrl

}

STATUS	current

DESCRIPTION	"A collection of objects providing extended device installation support."

::= { rcstGroups 4 }

===================================================================

-- object groups for rcst networking

===================================================================

rcstNetworkGroup OBJECT-GROUP

OBJECTS {

networkOamIpAddr,

networkOamIpNetworkMask,

networkLanIpAddress,

networkLanIpMask,

networkConfigFileDownloadUrl,

networkConfigFileUploadUrl,

networkLogFileUploadUrl

}

STATUS	current

DESCRIPTION	"A collection of objects providing basic networking management support."

::= { rcstGroups 5 }

rcstExtNetworkGroup OBJECT-GROUP

OBJECTS {

networkOamIpAddrAssign,

networkAirInterfaceDefaultGW,

networkNccMgtIpAddress

}

STATUS	current

DESCRIPTION	"A collection of objects providing extended networking management support."

::= { rcstGroups 6 }

rcstDnsGroup OBJECT-GROUP

OBJECTS {

networkDnsServerIpAddress

}

STATUS	current

DESCRIPTION	"IP address of the DNS server to be used to resolve URL's as needed."

::= { rcstGroups 7 }

===================================================================

-- object groups for return link

===================================================================

rtnConfigGroup OBJECT-GROUP

OBJECTS {

rtnconfDefIfLevel

}

STATUS	current

DESCRIPTION	"A collection of objects providing basic return link configuration support."

::= { rcstGroups 8 }

rtnExtConfigGroup OBJECT-GROUP

OBJECTS {

rtnconfMaxEirp

}

STATUS	current

DESCRIPTION	"A collection of objects providing extended return link configuration support."

::= { rcstGroups 9 }

rtnStatusGroup  OBJECT-GROUP

OBJECTS {

rcstRtnLinkStatus,

rtnstatusEbN0,

rtnstatusSFduration,

rtnstatusPayloadUnit

}

STATUS	current

DESCRIPTION	"A collection of objects allowing access to RTN link status."

::= { rcstGroups 10 }

===================================================================

-- object groups for forward link

===================================================================

fwdConfigGroup OBJECT-GROUP

OBJECTS {

fwdStartIndex,

fwdStartPopId,

fwdStartFrequency,

fwdStartPolar,

fwdStartFormat,

fwdStartModulation,

fwdStartRolloff,

fwdStartSymbolRate,

fwdStartInnerFec,

fwdStartRowStatus

}

STATUS	current

DESCRIPTION	"A collection of objects providing basic start forward link configuration support."

::= { rcstGroups 11 }

fwdStatusGroup  OBJECT-GROUP

OBJECTS {

fwdStatusIfIndex,

fwdStatusNetId,

fwdStatusCnr,

fwdStatusRowStatus

}

STATUS	current

DESCRIPTION	"A collection of objects providing basic forward link status."

::= { rcstGroups 12 }

fwdExtStatusGroup  OBJECT-GROUP

OBJECTS {

fwdStatusFormat,

fwdStatusFrequency,

fwdStatusPolar,

fwdStatusInnerFec,

fwdStatusSymbolRate,

fwdStatusRollOff,

fwdStatusModulation,

fwdStatusFecFrame,

fwdStatusPilot,

fwdStatusBer,

fwdStatusRxPower,

rcstFwdLinkStatus

}

STATUS	current

DESCRIPTION	"A collection of objects providing extended forward link status."

::= { rcstGroups 13 }

====================================================================

--    object groups for QOS

====================================================================

rcstQosGroup  OBJECT-GROUP

OBJECTS {

pktClassIndex,

pktClassPhbAssociation,

phbIdentifier,

phbName,

phbRequestClassAssociation,

requestClassIndex,

requestClassName,

requestClassChanId,

requestClassVccVpi,

requestClassVccVci,

requestClassCra,

requestClassRbdcMax,

requestClassRbdcTimeout,

requestClassVbdcMax,

requestClassVbdcTimeout,

requestClassVBDCMaxBacklog

}

STATUS	current

DESCRIPTION	"A collection of objects providing basic access to QOS configuration data."

::= { rcstGroups 14 }

rcstEnhancedClassifierGroup  OBJECT-GROUP

OBJECTS {

pktClassIpProtocol,

pktClassIpScrAddr,

pktClassIpScrAddrMask,

pktClassIpDstAddr,

pktClassIpDstAddrMask,

pktClassSrcPortLow,

pktClassSrcPortHigh,

pktClassDstPortLow,

pktClassDstPortHigh

}

STATUS	current

DESCRIPTION	"A collection of objects providing support for management of the enhanced classifier."

::= { rcstGroups 15 }

rcstMpegQosGroup  OBJECT-GROUP

OBJECTS {

requestClassPidPoolReference,

pidPoolIndex,

pidIndex,

pidValue,

pidPoolRowStatus

}

STATUS	current

DESCRIPTION	"A collection of objects providing access to MPEG related link QOS configuration data."

::= { rcstGroups 16 }

rcstAvbdcRepQosGroup  OBJECT-GROUP

OBJECTS {

requestClassAvbdcRepTime

}

STATUS	current

DESCRIPTION	"A collection of objects providing access to AVDBC repeat related QOS configuration data."

::= { rcstGroups 17 }

rcstGlobalQosGroup  OBJECT-GROUP

OBJECTS {

qosGlobalRbdcMax,

qosGlobalVBDCMax,

qosGlobalVBDCMaxBacklog

}

STATUS	current

DESCRIPTION	"A collection of objects providing access to global RCST QOS configuration data."

::= { rcstGroups 18 }

rcstGlobalAvbdcRepQosGroup  OBJECT-GROUP

OBJECTS {

qosGlobalAvbdcRepTime

}

STATUS	current

DESCRIPTION	"A collection of objects providing access to global AVDBC repeat related QOS configuration data."

::= { rcstGroups 19 }

rcstStrictQosGroup  OBJECT-GROUP

OBJECTS {

qosChannelIdStrictDispatching

}

STATUS	current

DESCRIPTION	"A collection of objects allowing management of strict channel ID dispatching."

::= { rcstGroups 20 }

===================================================================

-- object groups for rcst state

===================================================================

rcstStateGroup  OBJECT-GROUP

OBJECTS {

rcstMode,

rcstFaultStatus,

rcstLogUpdated,

rcstSwDownloadStatus,

rcstCurrentSoftwareVersion,

rcstAlternateSoftwareVersion,

rcstActivatedConfigFileVersion,

rcstDownloadedConfigFileVersion

}

STATUS	current

DESCRIPTION	"A collection of objects allowing access to RCST state."

::= { rcstGroups 21 }

===================================================================

-- object groups for rcst control

===================================================================

rcstControlGroup  OBJECT-GROUP

OBJECTS {

ctrlRebootCommand,

ctrlUserTrafficDisable,

ctrlCWEnable,

ctrlDownloadFileCommand,

ctrlUploadFileCommand,

ctrlActivateConfigFileCommand

}

STATUS	current

DESCRIPTION	"A collection of objects allowing basic RCST control."

::= { rcstGroups 22 }

rcstExtControlGroup  OBJECT-GROUP

OBJECTS {

ctrlRcstTxDisable,

ctrlOduTxReferenceEnable,

ctrlOduTxDCEnable,

ctrlOduRxDCEnable,

ctrlRcstLogonCommand,

ctrlRcstLogoffCommand

}

STATUS	current

DESCRIPTION	"A collection of objects allowing extended RCST control."

::= { rcstGroups 23 }

END=

# Security Considerations

Some of the readable objects in this MIB module (i.e., objects with a MAX-ACCESS other than not-accessible) may be considered sensitive or vulnerable in some network environments.

SNMP versions prior to SNMPv3 did not include adequate security. Even if the network itself is secure (for example by using IPsec), even then, there is no control as to who on the secure network is allowed to access and GET/SET (read/change/create/delete) the objects in this MIB module.

It is RECOMMENDED that implementers consider the security features provided by the SNMPv3 framework (see [RFC3410], section 8), including full support for the SNMPv3 cryptographic mechanisms (for authentication and privacy).

Further, deployment of SNMP versions prior to SNMPv3 is NOT RECOMMENDED.  Instead, it is RECOMMENDED to deploy SNMPv3 and to enable cryptographic security.  It is then a customer/operator responsibility to ensure that the SNMP entity giving access to an instance of this MIB module, is properly configured to give access to the objects only to those principals (users) that have legitimate rights to indeed GET or SET (change/create/delete) them.

# IANA Considerations

No action from IANA is needed.

# Acknowledgments

The authors thank Gorry Fairhurst for advice in the preparation of this document.

The authors recognize this document is a collective effort of the SatLabs Group (www.satlabs.org).

# References

## Normative References

[RFC2119]	Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997 (BEST CURRENT PRACTICE).

[RFC2578] McCloghrie, K., Ed., Perkins, D., Ed., and J. Schoenwaelder, Ed., "Structure of Management Information, Version 2 (SMIv2)", STD 58, RFC 2578, April 1999.

[RFC2579] McCloghrie, K., Ed., Perkins, D., Ed., and J.               Schoenwaelder, Ed., "Textual Conventions for SMIv2",               STD 58, RFC 2579, April 1999.

[RFC2580] McCloghrie, K., Perkins, D., and J. Schoenwaelder, "Conformance Statements for SMIv2", STD 58, RFC 2580, April 1999.

## Informative References

[ISO-MPEG] ISO/IEC DIS 13818-1:2000, "Information Technology;                 Generic Coding of Moving Pictures and Associated Audio Information Systems", International Standardisation Organisation (ISO).

[ETSI-DAT] EN 301 192, "Digital Video Broadcasting (DVB); DVB Specifications for Data Broadcasting", European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS] EN 301 421 "Digital Video Broadcasting (DVB); Modulation and Coding for DBS satellite systems at 11/12 GHz", European Telecommunications Standards Institute (ETSI).

[ETSI-RCS] ETSI 301 791 "Digital Video Broadcasting (DVB); Interaction Channel for Satellite Distribution Systems", European Telecommunications Standards Institute (ETSI).

[RFC3410] Case, J., Mundy, R., Partain, D., and B. Stewart, "Introduction and Applicability Statements for Internet-Standard Management Framework", RFC 3410, December 2002.

[RFC4259] Montpetit, M.-J., Fairhurst, G., Clausen, H., Collini-Nocker, B., and H. Linder, "A Framework for Transmission of IP Datagrams over MPEG-2 Networks", RFC 4259, November 2005.

[SATLABS]	SatLabs System Recommendations, version 2, December 2006. Available at www.satlabs.org.

# Authors' Addresses

Stephane Combes

ESTEC

European Space Agency

Keplerlaan 1 – P.O. Box 299

2200 AG Noordwijk ZH

The Netherlands

Email: stephane.combes@esa.int

URL:   telecom.esa.int

Petter Chr. Amundsen

VeriSat AS

P.O Box 1

1330 Fornebu

Norway

Email: pca@verisat.no

URL:   www.verisat.no

Micheline Lambert

Advantech Satellite Networks

2341 boul. Alfred-Nobel

Saint-Laurent (Montreal,)H4S 2A9

Quebec, Canada

Email: micheline.lambert@advantechamt.com

URL:   www.advantechsatnet.com

Hans-Peter Lexow

STM Norway

Vollsveien 21

1366 Lysaker

Norway

Email: hlexow@stmi.com

URL:   www.stmi.com

# Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any Intellectual Property Rights or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; nor does it represent that it has made any independent effort to identify any such rights.  Information on the procedures with respect to rights in RFC documents can be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any assurances of licenses to be made available, or the result of an attempt made to obtain a general license or permission for the use of such proprietary rights by implementers or users of this specification can be obtained from the IETF on-line IPR repository at http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any copyrights, patents or patent applications, or other proprietary rights that may cover technology that may be required to implement this standard.  Please address the information to the IETF at ietf-ipr@ietf.org.

# Disclaimer of Validity

This document and the information contained herein are provided on an "AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY, THE IETF TRUST AND THE INTERNET ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF  THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

# Copyright Statement

Copyright (C) The IETF Trust (2007).

This document is subject to the rights, licenses and restrictions

contained in BCP 78, and except as set forth therein, the authors  retain all their rights.

<<< RFC Ed Note: please remove the text below, prior to publications >>>

Change Log.

Rev -00

- First draft, for comment by the community.

Rev -01

- Second draft, for comment by the community.

The MIB module is renamed DVBRCS-MIB to reflect that it covers more than solely interface issues.

The MIB has been updated with

- a conformance section that captures the options

- structural changes and corrections to achieve this

- SYNTAX refinements where applicable

- UNITS declarations where applicable

Document reformatted to conform to I-D format conventions.

<<<End of RFC Ed Note >>