| Internet Engineering Task Force |  |
| --- | --- |
| Internet Draft |  |
| Document: draft--ipdvb-ule-txt |  |
| Category: Draft | 2003 |

- Encapsulation (ULE) for transmisf IP datagrams over MPEG-2/DVB networks

- Status of this Draft

This document is an Internet-Draft and is in full conformance with all provisions of Section 10 of RFC2026.

Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups. Note that other groups may also distribute working documents as Internet-Drafts. Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet- Drafts as reference material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at http://www.ietf.org/shadow.html.

- Document history

This draft is intended as a study item for proposed future work by the IETF in this area.  Comments relating to this document will be gratefully received by the authors)and the ip-dvb mailing list at: ip-dvb@erg.abdn.ac.uk

- Abstract

This document contains altra Lightweig(ULE) mechanism fof IP Datagrams directly over ISO ansport Streams (TS) as TS private dat. been widely accepted not only for providing digital TV services, but also as a subnetwork technology for building IP networks.

2. Conventions used in this document

4. SNDU Format

- Brid4.2 IPv4 Encapsulation

- 5. Processing at the Encapsulator and Receiver

- 5.2 Flushing the bitstream

- 6. Summary

8. Security Considerations

10. Authors' Addresses

This document describes an encapsulation for transport of IP datagrams or other network layer packets over ISO MPEG-2 Transport Streams [ISO-MPEG].  This is suited to services based on MPEG-2, for example the Digital Video Broadcast (DVB) architecture, the Advanced Television Systems Committee (ATSC) system [ATSC; ATSC-G], and other similar MPEG-2 based transmission systems. Such systems typically provide unidirectional (simplex) physical and link layer standards, and have been defined for a wide range of physical media (e.g. Terrestrial TV [ETSI-DVBT; ATSC-PSIP-TC], Satellite TV [ETSI-DVBS; ATSC-S],Cable Transmission [ETSI-DVBC; ATSC-PSIP-TC]).

Data for transmission over the MPEG-2 transport multiplex is passed to an encapsulator typically receives PDUs (Ethernet

Frames, IP datagrams or other network layer packets). It formats each PDU into a series of TS Packets (usually after adding an encapsulation header), which is sent over a TS logical channel.

The draft describes a mechanism aimed at a subset of the services supported by [draft-unisal-ipdvb-enc-01.txt].  The format of this document resembles this for ease of comparison and much of the background text is common, although the encapsulation protocol is different and more lightweight.

2. Conventions used in this document

ADAPTATION FIELD: An optionl variable-length extension field of the fixed-length TS Packet header, intended to convey clock references and timing and synchronization information as well as stuffing over an MPEG-2 multiplex [ISO-MPEG].

- AFC: Adaptation Field Control.

ATSC: Advanced Television Systems Committee [ATSC]. A set of framework and associated standards for the transmission of video, audio, and data, using the ISO MPEG-2 standard.

DSM-CC: Digital Storage Management Command and Control [ISO-DSMCC]. A formatting defined by the ISO MPEG-2 standard, which is carried in an MPEG-2 private section.

DVB: Digital Video Broadcast [ETSI-DVB]. A set of framework and associated standards for the transmission of video, audio, and data, using the ISO MPEG-2 standard.

ENCAPSULATOR: A network device which receives PDUs (Ethernet frames or IP datagrams) and formats these for output as a transport stream of TS Packets.MAC: Medium Access and Control of the Ethernet IEEE 802 standard of protocols.

- MPE: Multiprotocol Encapsulation [ETSI-DAT]. A scheme that

- encapsulates Ethernet frames or IP Datagrams, creating a

DSM-CC Section. The Section will be sent in a series of TS Packets

over a TS logical channel.

MPEG-2: A set of standards specified by the Motion Picture Experts Group (MPEG), and standardized by the International Standards Organisation (ISO) [ISO-MPEG].

- PES: Programme Elementary Scheme of MPEG-2 [ISO-MPEG].

PID: Packet Identifier. A field carried in the header of all MPEG-2 Transport Stream packets. This is used to identify the TS logical channel to which it belongs [ISO-MPEG].

PUSI: Payload_Unit_Start_Indicator of MPEG-2 [ISO-MPEG]. A PUSI value of zero indicates that the TS Packet does not carry the start of a new payload. In this document, a PUSI value of one indicates that the TS Packet does carry the start of a new payload.Wthe et to 1, in U indicates t pa one byte

PRIVATE SECTION: a syntactic structure used for mapping all service information (e.g. an SI table) into TS Packets.  A table may be divided into a number of sections.  All sections of a table must be carried over a single TS logical channel.

SI TABLE: Service Information Table. In this document, the term is used to describe any table used to convey information about the service carried in a TS Multiplex (e.g. [ISO-MPEG]). SI tables are carried in MPEG-2 private sections.

SNDU: Subnetwork Data Unit, an IPv4 or IPv6 datagram (or other subnetwork packet, e.g., an arp message or bridged Ethernet frame).

TS: Transport Stream [ISO-MPEG], a method of transmission at the MPEG-2 level using TS Packets; it represents level 2 of the ISO/OSI reference model. See also TS logical channel and TS Multiplex.

TS LOGICAL CHANNEL: a channel identified at the MPEG-2 level; it represents level 2 of the ISO/OSI reference model. All packets sent over a channel carry the same PID value

TS MULTIPLEX: A set of MPEG-2 transport stream channels sent over a single common physical link (i.e. a transmission at a specified symbol rate, FEC setting, and transmission frequency).

TS PACKET: A fixed-length 188B unit of data sent over an MPEG-2 multiplex [ISO-MPEG]; it corresponds to the cells, of e.g. ATM networks, and is frequently also referred to as a TS_cell.  Each TS Packet carries a 4B header, plus optional overhead including , encryption details and time stamp information to synchronise a set of Transport Streams.

3. Description of method

Routed IP packets and bridged Ethernet frames shall be encapsulated to form a Subnetwork Data Unit (SNDU). The method encapsulates IP packets, Ethernet frames or packets from other network protocols within a SNDU. The SNDU transmitted over an MPEG-2 transmission system, by placing it in the payload of a single TS Packet, or, if required, the SNDU may be fragmented into a series of TS Packets. Where there is sufficient space, the method allows a single TS Packet to carry more than one SNDU (or part there of).

All packets from a SNDU shall be assigned the same PID, and shall therefore form a part of the same logical TS channel.

In TS private streams, one may define the use of PUSI, AFC, aopriate [ISO-MPEG].. The meied in the 4 byte header of each TS Pack

The header of each TS Packet carries a one bit Payload Unit Start Indicator (PUSI) field whose semantics is defined for PES and PSI packets; for private data its use is not defined in the MPEG-2 standard Ibit is used by the encapthe start of a payload unit within the MPEG-2 TS Packet Payload. Hence, when used with this encapsulation, the following PUSI values are defined:

0 - the TS Packet does not contain the start of a SNDU but the continuation or end of a SNDU;

1 – the TS Packet contains the start of a SNDU.

The TS Packet Header also carries a two bit Adaptation Field Control (AFC) value, and TS packets from a ULE encapsulator will normally AFC value of '0allntrol field set to a value of '00'; in the case of a ull packet the value of the adaptation_field_control shall be set to '01'. The purpose of the adaptation field is primarily to carry timing and synchronisation information and occasionally to include stuffing bytes.

4. SNDU Format

The encapsulation format to be used for IP packets and bridged Ethernet frames is shown below:

| length | type |        SNDU field                | CRC-32 |
+---------------------------------------------------+-----+

- Figure 1: SNDU Encapsulation

Note that the SNDU field might include a “label” or some other form of discriminator field which in combination with the PID value, could be interpreted as a “Link-Level address”.

- Length Field

- A 16-bit value indicates the length in bytes of the SNDU (encapsulated Ethernet frame, IP datagram or other packet) counted from the byte following the type field up to and including the CRC. The special value 0x0000 indicates that there are no further SNDUs within the current TS packet (see section 5.1). The maximum value is 65531 bytes.

- Type Field

- The 16-bit type field indicates the type of payload being sent. Three types are suggested in this document. These are:

0x0800 : IPv4 Payload (according to IANA EtherTypes)

0x86DD : IPv6 Payload (according to IANA EtherTypes)

0x8847 : MPLS frame (according to IANA EtherTypes)0x????: ROHC Compressed IPv4 Packet

0x6558: Bridged Ethernet Frame (i.e. MAC header follows)

All other assignments should be coordinated with the values defined for IANA EtherTypes encapsulations.

- IPv4 SNDU

- The payload shall be a complete IPv4 datagram.

- IPv6 SNDU

- The payload shall be a complete IPv6 datagram.

- Bridged SNDU

- The payload shall be a bridged MAC frame (see section 4.1).

- CRC

- have a CRC fiela CRC value calculated by the encapsulator and checked by the receiver this protects the entire SNDU. A CRC-32 is re of this CRC is to protect an IP payload from undetected res errors duced by unexpecy also be used to detect the presence of uncorrected errors from the physical link (these hoin some casby other means).

- 4.1 Bridged Payload

The Bridged Payload shall carry an SNDU field with the following format shown in figure 2.

+-------------------------------+

|  Length and Type  fields (4B) |

+-------------------------------+

|  MAC destination address (6B) |

+-------------------------------+

|  MAC source address      (6B) |

+-------------------------------+

|  Ethernet (DIX) Type     (2B) |

+-------------------------------+

|                               |

|   (remainder of MAC frame)    |

|                               |

+-------------------------------+

|                   (4B) |

+-------------------------------+

- Figure 2: SNDU Format for a Bridged Payload

The MAC addresses shall be assigned according to the rules specified by the IEEE and may denote unknown, unicast, broadcast, and multicast link addresses. The type of frame shall be defined according to Ethernet [DIX].relate to the binding of M and are identified by the appropriate DIX Ethernet fra

In normal operation, it is expected that any padding appended to the Ethernet frame will be removed prior to forwarding. This requires the sender to be aware of such padding.

32-bit CRC

4.2 IPv4 Encapsulation

IPv4 datagrams will be transported over the MPEG-2 Transport Stream using the SNDU structure shown in figure 3.

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|   Length  (2B)        |     Type = 0x0800     |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|                                               |

|                IPv4 datagram                  |

|                                               |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|                   (CRC_32)                    |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

- Figure 3: SNDU Format for an IPv4 Datagram

IPv6 datagrams will be transported over the MPEG-2 Transport Stream using the SNDU structure shown in figure 4.

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|   Length  (2B)        |     Type = 0x86DD     |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|                                               |

|                IPv6 datagram                  |

|                                               |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

|                   (CRC_32)                    |

+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

- Figure 4: SNDU Format for an IPv6 Datagram

5. Processing at the Encapsulator and Receiver

5.1 Encapsulator processing

The encapsulator shall fragment the SNDU into a series of MPEG-2 TS Packets belonging to the same logical TS channel (figure 5).

+-----------------------------------------+

|Encap Header|       Subnetwork PU        |

+-----------------------------------------+

/         /                          \      \

/         /                            \      \

/         /                              \      \

+------*----------*  +------*----------*   +------*----------+

|MPEG-2| MPEG-2   |..|MPEG-2| MPEG-2   |...|MPEG-2| MPEG-2   |

|Header| Payload  |  |Header| Payload  |   |Header| Payload  |

+------+----------+  +------+----------+   +------+----------+

Figure 5: Encapsulation of a subnetwork Payload_Unit (SNDU) e.g. an IP datagram into a series of TS Packets (each TS Packet carries a header with a common Packet ID, PID, value).

The encapsulation layer will first wrap each payload unit (IP datagram or Ethernet frame) to form a SNDU. (This is similar to an AAL5 encapsulation in ATM networks). This SNDU will then be segmented into payload units of TS Packets; the resulting set of TS Packets will all use the same PID value and successive values for the continuation counter. This set will then be sent as a sequence over a TS multiplex or possibly as one burst over a satellite link.

Measurements of IP traffic have shown that the overwhelming majority of the datagrams have lengths of 1500 or 576 bytes for data and 40 or 48 bytes for control traffic; for example, these values represent almost 96% of the typical world-wide-web traffic.

The most frequent situations for the encapsulator are:

-  a short control packet using only a fraction of the

TS payload of 184 bytes;

-  a long data packet using a number of TS Packets with

the last one containing only a remainder.

Both of the most frequent values (576 and 1500) lead

to a fairly high overhead in the last TS Packet.

Since bandwidth is an expensive resource, as opposed to bandwidth in LANs, it is necessary to look for improvements. One possibility is header compression but this is outside the scope of this proposal. Another one is to pack SNDUs densely into TS Packets, i.e. whenever the encapsulator has more than one SNDU available it fills the TS Packets completely by appending the data of the following SNDU directly to the preceding one before segmentation.

When the encapsulator has not previously sent a TS Packet for the specified logical TS channel, or after an idle period, it will start sending the SNDU in the first available TS Packet.  This first TS Packet MUST carry a value of one in the payload_unit_start_indicator (PUSI) to indicate it contains the start of a SNDU.Payload Pointer value ate that the SN in the frava the TS Packet

The encapsulator will continue to fill subsequent TS Packets, until the end of the SNDU.

|   Subnetwork     |       |   Subnetwork     |

+------------------+       +------------------+

/          /               \        \   /          /

+------+--------+--------+----------+

start of | |Header| Pointer| SNDU 1 | SNDU 2   |

|              ^

+--------------+

- Figure 6 A TS Packet carrying the end of SNDU 1, followed by SNDU 2

If more packets are waiting at the encapsulator, acket has payload, it MAY start the nextvaie byte of the TS Packet payload. The PUSI MUST be set, if not already set. If the  set by thiload pointer MUST begned thepthe end of the first SNDU in t

+-------------+

- |      DU 3   |

\        \

\        \

+ |MPEG-2| end of | 0x0000 |  Unused  |

|    +------+--------+--------+----------+

PUSI=0            End

If the TS Packet carrying the final part of a SNDU has two or more bytes of unused payload, and an encapsulator does not start a new SNDU to use this unused payload, it MUST directly follow the final byte of the last SNDU with the value 0x00 00.  This value corresponds to a SNDU payload length of zero, and is an End Indicator, informing the receiver that there are no more SNDUs in this TS Packet payload.

When a SNDU is less than the size of a TS Packet payload, a TS Packet may be formed which carries a PUSI value of one and also an End Indicator.

MPEG-2 multiplexers do not usually flush their buffers, but store TS Packets until the buffer fills, assuming that the data comes in a more or less continuous stream. In case of data traffic, ths assumption no longer holds, leading to the problem that the last IP datagram will be only partly transmitted unless a special “push” packet is appended. This introduces additional overhead and is not an appealing solution.

A further SNDU within the same TS Packet is only started if it is already available in the encapsulators buffer at the time the previous one is encapsulated. The encapsulator does not wait for another SNDU to fill a TS Packet, because this would introduce additional latency.

5. eceiver Processing

TS Packet witnon-eSI vaicates ontains the start ofointer. The Payload Pointer value indicates that there are paeing reassembled at the start of the TS Packet payload. A

If there is either 0 or 1 byte of payload data remaining in the TS Packet after completion of the Current SNDU, the receiver MUST discard this remaining TS payload, and wait for the next TS Packet with the PUSI value set to 1.

If there more than one byte of payload data remains in the TS Packet after completion of the Current SNDU, the receiver MUST accept the next bytes as the start of the next SNDU (or an End Indicator), and continue processing the next SNDU.

- Payload Pointer Checking

A receiver that has partially received a SNDU (in the Current SNDU buffer) MUST also check the Payload Pointer, of any received packets with a PUSI value of 1.  A Payload Pointer value of 1 indicates that the receiver MUST discard any previously unreassembled SNDU, and start processing the new SNDU that directly follows the Payload Pointer.

A Payload Pointer value greater than 1, indicates the receiver MUST complete processing of the current SNDU, if there are ((Payload Pointer)-1) bytes remaining to complete the ANDU. If the receiver has not completed the current SNDU, and the Length Field indicates that it is awaiting a greater number of bytes than the ((Payload Pointer) –1), then the receiver has detected a delimiting error and the partially received SNDU MUST be discarded. The receiver MUST also discard ((Payload Pointer) –1) bytes prior to the first SNDU in the TS Packet, where it shall restart reassembly.

- Other Error Condistions

The receiver SHOULD also check the MPEG-2 Continuity Counter carried in the TS Packet Header.  This counter should increment by one for each TS Packet received on a logical TS channel. If the value is not incremented by one in successive packets (modulo 16), any partially received SNDU MUST be discarded. The receiver then waits for the next TS Packet with a PUSI value of 1.

The receiver  also heck the MPEG-2 Transport Error indicator carried in the TS Packet Header.  This flag indicates a transmission error for the logical TS channel. If the flag is set to a one, any partially received SNDU MUST be discarded. The receiver then waits for the next TS Packet with a PUSI value of 1.

6. Summary

This document defines a Encapsulation (ULE) to perform ent and flexible support for IPv4 and IPv6 network services over networks built upon the MPEG-2 Transport Stream (TS).

- 7. Acknowledgments

authored by: Horst D. Clausen, Bernharer, Hilmar Linmembers of the ip-dvb mailing list for the input provideicular, thmaommeceivedf

- 8. Security Considerations

- Security issues are not addressed in this document.

- 9. References

[ATSC] A/53, "ATSC Digital Television Standard", Advanced Television Systems Committee (ATSC), Doc. A/53, 1995.

[ATSC-DAT] A/90, "ATSC Data Broadcast Standard", Advanced Television Systems Committee (ATSC), Doc. A/090, 26 July 00

[ATSC-DATG] A/91, "Recommended Practice: Implementation Guidelines for the ATSC Data Broadcast Standard", Advanced Television Systems Committee (ATSC),Doc. A/91. 10 June 2001

[ATSC-G] A/54, "Guide to the use of the ATSC Digital Television Standard", Advanced Television Systems Committee (ATSC), Doc. A/54, 4 Oct 95

[ATSC-PSIP-TC] A/65A, "Program and System Information Protocol for Terrestrial Broadcast and Cable", Advanced Television Systems Committee (ATSC), Doc. A/65A, 23 Dec 1997, Rev. A – 31 May 2000

[ATSC-S] A/80, "Modulation and Coding Requirements for Digital TV (DTV) Applications  over Satellite", Advanced Television Systems Committee (ATSC), Doc. A/80, 17 July 99

[CLC99] Clausen, H., Linder, H., and Collini-Nocker, B., "Internet over Broadcast Satellites", IEEE Commun. Mag. 1999, pp.146-151.

[ETSI-DAT] EN 301 192 Specifications for Data Broadcasting, European Telecommunications Standards Institute (ETSI).

[ETSI-DVBC] EN 300 800 Digital Video Broadcasting (DVB); DVB interaction channel for Cable TV distribution systems (CATV), European Telecommunications Standards Institute (ETSI).

[ETSI-DVBS] EN 301 421 Digital Video Broadcasting (DVB); Modulation and Coding for DBS satellite systems at 11/12 GHz, European Telecommunications Standards Institute (ETSI).

[ETSI-DVBT] EN 300 744 Digital Video Broadcasting (DVB); Framing structure, channel coding and modulation for digital terrestrial television (DVB-T), European Telecommunications Standards Institute (ETSI).

[ISO-DSMCC] ISO/IEC IS 13818-6 Information technology -- Generic coding of moving pictures and associated audio information -- Part 6: Extensions for DSM-CC is a full software implementation, International Standards Organisation (ISO).

[ISO-MPEG] ISO/IEC DIS 13818-1 Information technology -- Generic coding of moving pictures and associated audio information: Systems, International Standards Organisation (ISO).

[ISO-VID] ISO/IEC DIS 13818-2 Information technology -- Generic coding of moving pictures and associated audio information: Video, International Standards Organisation (ISO).

[ISO-AUD] ISO/IEC 13818-3:1995 Information technology -- Generic coding of moving pictures and associated audio information -- Part 3: Audio, International Standards Organisation (ISO).

[LLC] IEEE Logical Link Control (ANSI/IEEE Std 802.2/ ISO 8802.2), 1985

[RFC3077] E. Duros, W. Dabbous, H. Izumiyama, Y. Zhang, "A Link Layer Tunneling Mechanism for Unidirectional Links", RFC3077.

[RFC3095] C. Bormann, et al, "RObust Header Compression (ROHC):

Framework and four profiles: RTP, UDP ESP and uncompressed", RFC3095.

[SI-DAT] SI-DAT group, Second Draft DVB Specification for Data Broadcasting" Geneva, 15 Jan. 1997

10.Authors' Addresses

- Godred Fairhurst

- Department of Engineering

University of Aberdeen

Aberdeen, AB24 3UE

UK

Email: gorry@erg.abdn.ac.uk

- Web: http://www.erg.abdn.ac.uk/users/gorry

- Bernhard Collini-Nocker

University of Salzburg

5020 Salzburg

Email: [bnocker]@cosy.sbg.ac.at

- Full Copyright Statement

"Copyright (C) The Internet Society (date). All Rights Reserved. This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works. However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the Internet Society or other Internet organizations, except as needed for the purpose of developing Internet standards in which case the procedures for copyrights defined in the Internet Standards process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the Internet Society or its successors or assigns.

- . IANA Considerations

- This document will require IANA involvement.

The payload type field defined in this document must be aligned with an existing IANA registry or the following values need to be assigned by the IANA:

- Payload Type Field